SAFETY DATA SHEET

## SECTION 1. PRODUCT AND COMPANY IDENTIFICATION

MANUFACTURER: TRANSENE COMPANY, INC.

# ADDRESS: DANVERS INDUSTRIAL PARK

# 10 ELECTRONICS AVENUE DANVERS, MA 01923,

# TEL: (978) 777-7860 FAX: (978)-739-5640

# WWW.TRANSENE.COM

#### EMERGENCY NO. 1-800-424-9300 CHEMTREC

| MATERIAL NAME: COPPER ETCHANT APS-100 REVISED: September 2013 #### General Use: Etchant for copper and copper films. #### Product Number:060-060APS100 |  |  |  |  |
| --- | --- | --- | --- | --- |
| ###### SECTION 2. HEALTH HAZARD INFORMATION ###### GHS Classifications H272: Oxidizing liquids : Category 2 H315: Skin corrosion / Skin irritation : Category 2 H319: Serious eye damage / Eye irritation : Category 2A Pictograms or Hazard symbols |  |  |  |  |
| Danger: May intensify fire. Oxidizer. Warning: Harmful if swallowed, in contact with skin, or inhaled. Causes skin irritation and serious eye irritation. Precautionary Statements P210 Keep away from heat, flames, and hot surfaces. No smoking. P220 Store away from combustible materials. P221 Take any precaution to avoid mixing with combustibles. P264 Wash thoroughly after handling. P280 Wear protective gloves, clothing, and eye and face protection. P302 + P352 If on skin, wash with plenty of water. Remove contact lenses if present and easy to do so. Continue rinsing. P305 + P351 + P338 If in eyes, rinse cautiously with water for several minutes. P332 + P313 If skin irritation occurs, get medical advice/attention. P337 + P313 If eye irritation persists, get medical advice/attention. P362 + P364 Take off contaminated clothing and wash it before reuse. P370 + P378 In case of fire use water spray or fog, carbon dioxide, or dry chemical to extinguish. P501 Dispose of contents/container in accordance with local/regional/national/international regulations. |  |  |  |  |

###### SECTION 3.COMPOSITION/INFORMATION ON INGREDIENTS

| Material Ammonium peroxydisulfate CAS# 7727-54-0 Water CAS# 7732-18-5 Total | Wt % 15-20 80-85 100 |  |
| --- | --- | --- |

SECTION 4. FIRST AID MEASURES

#### EFFECTS OF OVEREXPOSURE

FIRST AID:

Eye Contact: May cause irritation and pain.  Seek medical attention.

Skin Contact: Corrosive to exposed skin. Flush skin well with water for 15 minutes. Remove affected clothing.

Inhalation:  May irritate the mucous membranes.  May cause lung edema.  Symptoms may include throat irritation, shortness of breath, inflammation of nasal passages, and coughing.  Asthma-like symptoms may develop.

Ingestion:   May cause abdominal pain, nausea, and vomiting.

| SECTION 5. FIRE FIGHTING MEASURES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Flash Point and Method non-flammable | # Autoignition Temp. NA | #### Flammability Limits In Air | LOWER # NA | UPPER # NA |  |

Extinguishing media: Water spray or fog, carbon dioxide and dry chemical.  Do not use organic media.

Special fire fighting procedures: Wear chemically retardant gear and NIOSH approved self-contained breathing apparatus. Oxidizer.  Heat of reaction with combustibles or reducing agents may enhance fire.

Explosion hazard when mixed with finely powdered organic matter, metal powder, or reducing agents.

SECTION 6. ACCIDENTAL RELEASE MEASURES

SPILLS, LEAKS: Ventilate area of leak or spill. Remove sources of ignition.  Clean-up personnel should wear protective clothing and NIOSH approved respirator if no ventilation is available. Dike and cover the contaminated areas with absorbent, non-combustible material such as earth, sand, or vermiculite.

#### SECTION 7. HANDLING AND STORAGE

Wash thoroughly after handling.  Remove contaminated clothing and wash before re-use.  Do not breathe dust, mist, or vapor.  Do not expose eyes, skin, or clothing.  Keep container closed tightly.  Avoid contact with combustibles.  Do not use with metal tools or items.  Use with adequate ventilation or respiratory protection. Do not store near combustibles or in direct sunlight.  Store in a cool, dry, well-ventilated area away from incompatible substances.  Separate from metals, reducing agents, and organics.  Empty containers may be hazardous.

SECTION 8. EXPOSURE CONTROL/PERSONAL PROTECTION

Respiratory protection: Wear NIOSH/MESA approved full or half face piece (with goggles) respiratory protective equipment to avoid exposure to mist. A respiratory protection program complying with requirements of 29CFR 1910.134 is recommended.

Ventilation:	Where adequate ventilation is not available, use NIOSH approved vapor respirator with dust, fume and mist filters. Local ventilation through fume hoods or laminar flow stations is also preferred. Keep fumes away from strong bases.

Protective gloves: Skin contact should be minimized through use of rubber gloves.

Other protective equipment:	Steel tipped shoes/eye wash station/chemical safety chemical retardant clothing.  Eye protection: Safety goggles / face shield

### SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES

Form :								  Liquid

Appearance :                                                                           Colorless

Odor :                                                                 Acrid

pH :                                                                                          < 1

Melting point:                                                                          Not available

Boiling point/Boiling range :                                                 100 oC (water)

Flash point :                                                                             Non-flammable.

Ignition point :                                                                         Will not ignite.

Danger of explosion:                                                               Product is not explosive

Decomposition temperature:                                                 > 150 oC

Vapor density (Air = 1) :                                                        Not available

Volatiles, %:                                                                            80-85

Vapor pressure at 15o C, mm Hg:                                         51 mm Hg at 25 oC

Specific gravity :                                                                     1.08 g/cc

Solubility in / Miscibility:                                                       Completely miscible in water

Evap. Rate (Water = 1):                                                          < 1

##### SECTION 10.  STABILITY AND REACTIVITY

| Stability |  | Stable X Conditions to avoid: Excess heat , light, confined spaces Unstable |  |
| --- | --- | --- | --- |
| Incompatible with: Metals/metal powders, reducing agents, strong bases, acetic acid, alcohol, acetone, aniline, hydrogen sulfide, carbides, organic solvents, combustibles, chromic acid, flammables, cyanides, sulfides. Hazardous decomposition products: oxygen, ozone. Burning may produce nitrogen oxides, sulfur oxides, and sulfuric acid. |  |  |  |
| Hazardous polymerization: | May occur Conditions to avoid: Excess heat, damp. Will not occur X |  |  |

SECTION 11. TOXICOLOGICAL INFORMATION ACUTE: LD50 (oral, rat): 3,500 mg/kg Carcinogenicity: no SECTION 12. ECOLOGICAL INFORMATION Bioaccumulation : There is no evidence of bioaccumulation. Ecotoxicity : No data SECTION 13. DISPOSAL CONSIDERATIONS DISPOSAL: Dispose of in accordance with all federal state and local regulations. Send waste to an approved waste disposal facility. SECTION 14. TRANSPORTATION INFORMATION Class 5.1 PG III UN3139 Shipping Name: Oxidizing Liquid, N.O.S. (Ammonium Persulfate Solution) SECTION 15. REGULATORY Symbol: O, oxidizing R-Phrases: R8: Contact with combustible material may cause fire R20/21/21: Harmful by inhalation, in contact with skin and if swallowed R41: Risk of serious damage to eyes S-Phrases: S17: Keep away from combustible material. S23-36/37/39-45 Do not breathe vapor. In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. Wear suitable protective clothing, gloves, and eye/face protection. In case of accident or if you feel unwell, seek medical advice immediately (show the label where possible). SECTION 16. OTHER INFORMATION NFPA Codes: Health: 2 Flammability: 0 Reactivity: 3 All ingredients of this product are listed on the US TSCA inventory under their parent anhydrous compounds.